Copyright ã 1995, 1996  Mark Stockman    (Double

Washington State University

Department of Physics

Physics 101-3

Lecture 13

Chapter 9-1 through 9-5

Statics: Bodies in Equilibrium

Contents

## 1. Conditions of Equilibrium

Equilibrium implies that a body and any part of it are at rest (in the corresponding inertial system). For a body to be in equilibrium it is necessary that both the total force and total torque vanish,

F=0,   t=0,

called the first and the second conditions of equilibrium.

Here, F is the total external force, and t is the total external torque.

Each component of the force and torque about each axis should vanish.

The above conditions are necessary, but sometimes not sufficient. In most circumstances, these conditions allow to solve problems. In some rare cases, the statics problems do not have solution at all (everything is determined by the deformations in the system).

## 2. Examples

Example 1: Construction crane:

Find tension in the upper (red) cable for m=1000 kg and q=15o.

Solution: We find and equate to zero the total torque on the lever.

t=lFTsinq−lmg=0.

From this, we immediately find,

FT=mgsinq=1000 kg ⋅9.80ms2sin15o=38,000 NThus, the tension in the cable significantly exceeds the weight of the load, 9800 N.

mg

Example 2: Ladder against a frictionless wall.

Find all forces. If the coefficient of friction with the ground is ms=0.4, find whether the equilibrium is possible.

Solution: We use the first condition of equilibrium for the y components of the forces,

Fy=-mg+FGy=0   ⇒   FGy=mg.

Then we use the first condition for the x components,

Fx=FW+FGx=0   ⇒   FGx=-FW

Finally, we are using the second condition,

FT

t=-FWl2sinQ+FGxl2sinQ+FGyl2cosQ+mg⋅0⋅cosQ=0.

We know that FGx=-FW and FGy=mg. Substituting these, we obtain

2FWsinQ=mgcosQ   ⇒FW=mg2tanQ

Summarizing the results,

FW=mg2tanQ,   FGx=-FW,   FGy=mg

Substituting the numerical data,

FW=20kg⋅9.80ms22tan25o=210 N,   FGx=-210 N,   FGy=20kg⋅9.80ms2=200 N.

Consider now stability with respect to sliding. Because FGx is due to the friction,

FGx<msmg   ⇒mg2tanQ<msmg     ⇒tanQ>1ms

We have ms=0.4, correspondingly, the condition is

tan25O>10.4,   or   0.47>2.5.

This inequality is false. Thus, our ladder is very unstable! To be stable, the angle should have been

tanQ=2.5,   or   Q>68O.

Similarly, for the text’s example,

m=12.0 kg,  Q=Arcsin4/5=51.1o. Substituting these, we obtain

FW=12kg⋅9.80ms22tan53o=44 N,   FGx=-44 N,   FGy=12kg⋅9.80ms2=118 N.

This precisely agrees with the text. By the way, that ladder is also unstable.

Double-click to activate the demonstration package (requires Interactive Physics II by Knowledge Revolution)

## 3. Stability

Double-click to activate the demonstration package (requires Interactive Physics II by Knowledge Revolution)

Stability of Ships